Interim Progress Report

Project Title

Ocean Wind Field Estimation Using Sentinel-1 SAR Imagery for Offshore Wind Farm Planning

Student Details

Name: Dipanshu
Roll Number: 25113043

1. Project Overview

The objective of this project is to develop an automated workflow for estimating ocean surface wind fields using Sentinel-1 Synthetic Aperture Radar (SAR) imagery over Indian coastal regions. The retrieved wind vectors will ultimately be used for offshore wind resource assessment and wind farm planning.

The final goal is to build an API-based system where a user provides an Area of Interest (AOI) and a date range, and the system returns the corresponding wind field vectors for the selected coastal region.

The present work focuses on establishing the data acquisition and preprocessing pipeline required for wind retrieval.

2. Study Area

The selected study area for this work is the Gujarat coastline due to its high offshore wind energy potential.

A custom Area of Interest (AOI) polygon covering the Gujarat coastal region was defined using geographic coordinates and integrated into Google Earth Engine (GEE).

3. Work Completed

3.1 Sentinel-1 Data Acquisition

A Google Earth Engine workflow was developed to automatically retrieve Sentinel-1 Ground Range Detected (GRD) imagery.

The implemented workflow performs:

- AOI-based image filtering

- Date-range filtering

- Instrument mode selection (IW)

- VV polarization extraction

- Image compositing

The following date range was tested:

- Start Date: 04 May 2026

- End Date: 04 June 2026

This confirms successful access to Sentinel-1 SAR data for the selected coastal region.

3.2 SAR Data Preprocessing

Radiometric Calibration

Sentinel-1 GRD products available through Google Earth Engine are already radiometrically calibrated. Therefore, the VV backscatter values were directly extracted for further analysis.

Speckle Noise Reduction

SAR imagery inherently contains speckle noise caused by coherent radar scattering.

A median filtering approach was implemented to reduce speckle while preserving major ocean surface features.

Image Export

The processed Sentinel-1 VV image was successfully exported as a GeoTIFF file and loaded into Python for subsequent analysis.

3.3 Land Masking

Since wind retrieval is only meaningful over ocean surfaces, a land masking procedure was implemented.

The Global Surface Water dataset available in Google Earth Engine was used to generate a sea mask.

The resulting workflow:

- Identifies water pixels.

- Removes land pixels.

- Retains only offshore regions.

This step significantly improves the quality of subsequent wind retrieval operations by eliminating terrestrial backscatter effects.

3.4 Python Processing Pipeline

A Python-based workflow was developed in Google Colab for:

- Reading GeoTIFF imagery

- Handling missing values

- Performing preprocessing operations

- Preparing SAR patches for future wind retrieval models

The workflow successfully loads and processes Sentinel-1 imagery exported from Google Earth Engine.

3.5 Investigation of Wind Direction Retrieval Using ResNet

A review of the research paper:

"Wind Direction Retrieval from Sentinel-1 SAR Images Using ResNet"

was conducted to understand deep-learning-based wind direction estimation.

Initial experiments were performed to load a ResNet model and test inference procedures.

During model inspection it was found that the available checkpoint corresponded to a standard ImageNet-trained ResNet model with 1000 output classes rather than a wind-direction retrieval model.

Therefore, wind direction retrieval using ResNet has not yet been completed and remains under investigation.

4. Results Obtained So Far

The following milestones have been successfully achieved:

✓ Gujarat coastal AOI defined

✓ Sentinel-1 imagery retrieval automated

✓ Google Earth Engine integration completed

✓ GeoTIFF export pipeline established

✓ Speckle filtering implemented

✓ Land masking implemented

✓ Python processing workflow developed

✓ Initial investigation of ResNet-based wind direction retrieval completed

5. Current Challenges

The primary challenge identified during the interim phase is the availability of a suitable pretrained deep-learning model for wind direction estimation.

The ResNet architecture described in the literature requires weights trained specifically on Sentinel-1 SAR imagery, which are not publicly available.

As a result, alternative approaches for wind direction estimation are currently being evaluated.

6. Future Work

The remaining tasks for the project are:

6.1 Wind Direction Retrieval

Two approaches will be explored:

Option 1: Deep Learning Approach

- Train or obtain a ResNet-based wind direction model

- Generate directional estimates directly from SAR imagery

Option 2: Auxiliary Meteorological Data

- Retrieve wind direction from ERA5 reanalysis products

- Use the retrieved direction as an input to the wind inversion model

6.2 CMOD5.N Wind Speed Retrieval

Implementation of the CMOD5.N Geophysical Model Function (GMF) to estimate:

- Surface wind speed at 10 m height (U10)

- Wind vector magnitude

using:

- SAR backscatter (σ0)

- Incidence angle

- Wind direction

6.3 Wind Vector Generation

Generation of:

- Wind speed maps

- Wind direction maps

- Wind vector fields

for the Gujarat offshore region.

6.4 Validation

Validation of retrieved wind fields using:

- ERA5 reanalysis products

- Available meteorological observations

Performance metrics such as RMSE and correlation coefficient will be computed.

6.5 API Development

Development of a user-facing API where:

Input:

- Area of Interest

- Date Range

Output:

- Wind speed

- Wind direction

- Wind vector field

7. Conclusion

Significant progress has been made toward establishing an end-to-end offshore wind retrieval workflow using Sentinel-1 SAR imagery.

The data acquisition, preprocessing, land masking, and Python processing stages have been successfully implemented. Preliminary exploration of deep-learning-based wind direction retrieval has also been completed.

The next phase of the project will focus on wind direction estimation, CMOD5.N inversion, wind vector generation, and API integration, ultimately leading to an automated offshore wind resource assessment framework.